Attachments: Bill of Materials

Bill of Materials

| SL No. | Name | Specifications | Quantity | Dimension |
| --- | --- | --- | --- | --- |
| 1 | 7 sensor array- Analog | - Input Voltage- 3.3-5.0 V - Input Current ~100 mA - Output Signal Type- Analog - Reverse Polarity Protection- Yes - Dimensions- 100*15*3 mm - Channel – 7 - From Robojunkies - Weight- 5 grams | 1 | 100mm * 15mm * 3mm |
| 2 | N20 12V 300RPM Micro Metal Gear Motor with Encoder | - Voltage – 12v - With Encoder - Count: 3PPR (Pulse per Revolutions). - 300 RPM | 3 | 24 x 12 x 10 mm. |
| 3 | Motor Driver TB6612FNG | - Operating Voltage (VDC): 15 - Peak Current (A): 3.2 - Continuous Current (A): 1.2 | 2 | 30 x 20 x 10 mm |
| 4 | Arduino Nano | - Using Atmel Atmega328P-AU MCU - Support for external 5V ~ 12V DC power supply - 14 Digital input / output ports: TX, RX, D2 ~ D13 - 1 pair of TTL level serial transceiver ports RX / TX | 1 | 45x18x5 mm |
| 5 | Flipo 1000mAh 3S 11.1V 30C Li-Po Battery | - Model No.: FLIPO 1000/3S-30C - Capacity (mAh): 1000 - Weight (gm): 85 - Output Voltage (VDC): 11.1 - Charge Rate (C): 1 ~ 3 - Discharge Plug: XT-60 - Balance Plug: JST-XH - Dimensions: 19.5x34x72mm - Max. Burst Discharge (C): 60 C (60 A). - Max. Charge Rate: 5 C - Max. Continuous Discharge: 30 C (30 A) | 2 | 19.5x34x72mm |